# Framing, Preprocessing & Synchronization

Architect's Design — Standalone Reference

A clean-slate design for the three layers underneath the model pipeline — written to stand on its own, elaborated in full, and corrected against how coach assignment actually works: gap detection on the OCR camera, not a generic presence signal.

Scope: Capture policy, preprocessing tasks, coach-boundary assignment, dashboard hierarchy   Input: 7 genlocked cameras, mp4 per pass   Updated: 2026-07-11

# 0. Overview

One decision drives everything below: input arrives as genlocked mp4 files, one complete recording per camera per train pass — not a live per-frame socket. That makes synchronization a batch manifest-building problem: build one correct, complete record of what happened, once, before any model runs — not a live-stream timeout/join problem where you're guessing whether a message is still coming.

The other load-bearing fact, corrected in this version: coach boundaries aren't inferred from a vague “presence signal.” The OCR camera itself detects the physical gap between two coaches and timestamps it. Everything captured between two consecutive gap timestamps — from any of the 7 cameras — belongs to whichever coach number the OCR camera read in that same window. That's the actual assignment rule, and it's simpler and more auditable than anything inferred statistically.

# 1. Framing

Three extraction policies, one per camera role, all reconciled against the shared genlock clock. Framing isn't “pick a frame rate” here — each role has a different physical reason for its own policy, and a dropped frame is accounted for explicitly rather than assumed away.

All 3 lanes share the same shape — integrity check, parse genlock index, role-specific extraction, stamp envelope, write ledger. They differ only in the amber policy box.

### Walkthrough

- Files land per pass. Each of the 7 cameras writes its own mp4 for a train pass. Nothing starts until all 7 are present and readable.

- Integrity check first, always. A truncated or corrupt file (power blip, storage hiccup — normal in a rail yard, not exceptional) gets quarantined and alerted immediately, not discovered three stages later when a model chokes on it.

- Genlock index is the join key. Every frame's physical index and embedded timestamp is read from the container before anything else happens — this is what every later synchronization step keys off, not wall-clock time.

- Extraction policy differs by physical role, not preference. OCR only needs the plate read once — trimming empty roll saves calls. Component cameras must sample at an identical stride or the 4-way tile desyncs geometrically. Underbogie needs near-every-frame because a part passing underneath doesn't get a second look.

- Every frame is stamped with a full identity envelope before it goes anywhere else — camera, index, timestamp, session. This is what lets synchronization later be a lookup, not a guess.

- Ledgers record what actually happened, not what was expected — declared frame count vs. actual extracted count, with gap ranges explicit. This is the raw material the coach-assignment step in §3 depends on.

- The barrier only waits on completeness, not on cross-camera agreement — genlock already guarantees agreement; the barrier just confirms all 7 ledgers exist before declaring the pass ready to process.

| Role | Extraction policy | Why |
| --- | --- | --- |
| OCR (1 cam) | Presence-gated, native fps | Plate only needs to be read once clearly — trimming empty roll saves OCR calls on frames with nothing to read |
| Component (4 cams) | Fixed stride, identical across all 4 | Tiling assumes 4 simultaneous frames — any stride mismatch between cams desyncs the composite geometrically, not just temporally |
| Underbogie (2 cams) | Near-every-frame | Components pass directly beneath at speed with no second look — under-sampling here is unrecoverable |

# 2. Preprocessing

Three stages apply to every frame regardless of source, then each camera role gets its own specialized chain. Two things are easy to skip in a first build — geometric calibration and low-light handling — and both are exactly what the MVIS spec's measurement-based defects (buffer height in mm, wheel-shelling depth in mm) depend on.

Crit dashed box = a physics limit, not an engineering gap. Warn = flagged but non-blocking. Trunk above the fork is the only preprocessing shared by all 7 cameras.

### Walkthrough

- Quality gate runs before anything expensive. Blur, brightness, occlusion — reject here, and every downstream stage (undistort, tile, model inference) never has to touch a useless frame.

- Undistortion is not optional. Every camera has lens distortion. Skip correction and every pixel-based measurement downstream — buffer height, wheel-shelling depth — is measuring a warped image, not the real part.

- Photometric normalization gets 7 cameras onto one distribution. Different mounts, different ambient light — models trained once need consistent input, not 7 different lighting conditions arriving unannounced.

- The fork happens only after the shared corrections — each role gets exactly what it needs: OCR gets text-specific enhancement, Component gets zone-cropped before tiling, Underbogie gets low-light handling that's always on, not conditional, because the pit is never daylight.

- The seam-consistency check doesn't fix anything — it flags. A lighting mismatch between two tiles usually means a dirty lens or a drifting mount, worth surfacing as a maintenance signal, not silently averaged away.

- Wheel-shelling depth is called out as a hardware limit, not a software gap. A single 2D RGB camera cannot reliably resolve sub-2mm surface depth. This defect class needs a different sensor (laser line scanner or structured light), or should be marked out-of-scope for the current rig.

| Stage | Applies to | Task | Why it's here |
| --- | --- | --- | --- |
| Common | All 7 cams | Quality gate — blur / brightness / occlusion | Rejects unusable frames before any GPU cycle is spent |
| Common | All 7 cams | Geometric undistortion (per-camera calibration matrix) | Required for every mm-based measurement in the MVIS spec |
| Common | All 7 cams | Photometric normalization | One consistent input distribution, not 7 |
| OCR | 1 cam | ROI crop + grayscale/CLAHE/sharpen | Plate legibility is the only goal here |
| Component | 4 cams | Per-cam zone crop → tile → seam-consistency flag | Correcting each camera before tiling makes the composite trustworthy |
| Underbogie | 2 cams | Low-light gain → aggressive undistort → perspective correction | Pit environment is never daylight — the only mode, not a branch |

# 3. Coach Assignment & Synchronization

This is the corrected mechanism. The OCR camera does two jobs at once: it reads the plate number, and it watches for the physical gap between two coaches — the brief moment where no coach body is in frame. Every gap gets a timestamp. Everything captured between two consecutive gap timestamps, from any of the 7 cameras, belongs to whichever coach number was read in that same window.

Warn dashed = the two places this design admits it could be wrong (genlock drift, camera gaps) and handles explicitly. Crit = a mismatch worth stopping for, not silently absorbing.

### Walkthrough

- The OCR camera is the boundary detector, not just the reader. It watches continuously across the whole pass. When no coach body or plate is visible for a short run of frames, that's a gap, and it gets a timestamp the moment it's detected.

- Two consecutive gaps define one coach's window. Everything the OCR camera reads between gap[i] and gap[i+1] belongs to one coach. The first coach's window starts at session start; the last ends at session end.

- The coach number comes from a vote, not a single read. Take the highest-confidence or majority OCR read across all frames in the window as that window's coach_id.

- Sanity-check against the known consist. Compare the number of detected windows to the train's expected coach count. A mismatch means a missed or falsely-triggered gap — flag it for a human rather than silently mis-assigning components.

- Genlock does the rest of the work for free. Once a coach's window is defined in the OCR camera's frame-index space, the identical range applies directly to all 6 other cameras — no separate boundary detection per camera.

- Coverage gaps are still checked per camera, per window — genlock guarantees timing alignment, not that every camera actually captured every frame.

- The Coach Envelope is the one object everything downstream reads. Built once, deterministically, before any model runs — coach_id, frame range, per-camera completeness, calibration profile.

| Case | Handling |
| --- | --- |
| First / last coach has no gap on one side | Window bounded by session start / session end instead of a Gap Marker |
| Two coaches merged into one window (missed gap) | Caught by the consist-count check — flagged for manual review, not silently assigned |
| False gap (glare, reflection, camera blip) | Same consist-count check catches an over-count of windows |
| OCR reads conflict within one window | Majority-vote / highest-confidence resolves it; a genuinely split vote gets flagged low-confidence |
| Camera drops frames mid-window | Per-camera completeness check inside the window, independent of coach assignment itself |

# 4. Dashboard Data Hierarchy

The Coach Envelope is the engineering object. What a station operator actually looks at is simpler: Train number → Coach number → Components (Undercarriage + Bottom). This is that hierarchy, built directly from the coach-assignment mechanism above and the defect taxonomy already established.

⇄ marks the ~7 cross-domain defect classes — they show up under both buckets because they genuinely need both camera views.

### Walkthrough

- Train number is the root — read once from the OCR camera, shared by every coach in the pass.

- Coach number comes straight from the gap-window mechanism in §3 — each coach the operator sees is exactly one Coach Envelope, no separate lookup needed.

- Two buckets under each coach, matching what an inspector actually walks under and around — Undercarriage (what the 2 underbogie cameras saw) and Bottom/Component (what the 4 tiled component cameras saw).

- Cross-domain defects appear in both buckets, marked, not hidden. An operator searching “brake” under Undercarriage should find it — and the same entry under Bottom should say so plainly.

- Leaf level is the actual actionable unit — a defect entry with a thumbnail, a confidence score, and a confirm/dismiss action, not a raw model output.

# 5. Pros and Cons, for This System Specifically

|  | Point | Why it matters here |
| --- | --- | --- |
| Pro | Coach assignment is deterministic and auditable | A gap timestamp either exists or it doesn't — no statistical inference |
| Pro | No timeout guessing anywhere in the sync path | The manifest is complete and correct before any model runs |
| Pro | Genlock does cross-camera alignment for free | One boundary detection (on the OCR camera) instead of 7 independent ones |
| Pro | Durable audit trail by construction | The Coach Envelope + Gap Markers are themselves the record of what was seen and when |
| Con | Not real-time | Whole file must land before the manifest builds — no mid-pass alerting with this design |
| Con | Consist-count check needs a known consist | Without an expected-coach-count reference, a missed or false gap won't be caught automatically |
| Con | Bet entirely on genlock holding | Mitigated by the drift-check, but the whole design assumes hardware sync stays correct for years |
| Con | Gap detector itself needs tuning | Too strict = false-split on small couplings; too loose = two coaches merge into one window |

Companion to the main pipeline architecture doc — see the 7-camera topology and MVIS defect taxonomy sections there for the full component/defect list this hierarchy is drawn from.